Biocompatibility Testing Matrix

Volumetric Airflow Gauge

Team Members:

Matthew Chakan

Justin Kiswardy

Michael Nilo

Principal Investigator:

Guy Guimond

Submittal Date:

April 17, 2007

The volumetric airflow gauge (VAG) is a gauge that is intended to be incorporated into a standard BVM system, providing the user with a constant display of the air volume administered to the patient being treated.  The casing for the device will be constructed of materials similar to those found in standard resuscitation equipment, therefore, new biocompatibility issues will be minimal at best.

The VAG is not intended to be in direct contact with the patient, however, the device will be handled by the resuscitator.  Therefore, in accordance with ISO 10993, the volumetric airflow gauge must comply with tests intended for devices with limited skin contact (<24 hrs.).  The tests will include cytotoxicity, sensitization, and irritation the user may experience as a result of using the device.  The tests to be performed are indicated in table 1.

Table 1. Biocompatibility test matrix, based on FDA document ISO 10993

| Device Categories |  |  | Initial Evaluation |  |  |  |  |  |  |  | Supplemental Evaluation |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Body contact | Contact duration | Cytotoxicity | Sensitization | Irritation | Systemic toxicity | Subchronic toxixity | Genotoxicity | Implantation | Hemocompatibility | Chronic toxicity | Carcinogeniety |
| Surface devices | Skin | ≤ 24 hours | x | x | x |  |  |  |  |  |  |  |
|  |  | 24 hours to 30 days |  |  |  |  |  |  |  |  |  |  |
|  |  | > 30 days |  |  |  |  |  |  |  |  |  |  |
|  | Mucosal membrane | ≤ 24 hours |  |  |  |  |  |  |  |  |  |  |
|  |  | 24 hours to 30 days |  |  |  |  |  |  |  |  |  |  |
|  |  | > 30 days |  |  |  |  |  |  |  |  |  |  |
|  | Breached surface | ≤ 24 hours |  |  |  |  |  |  |  |  |  |  |
|  |  | 24 hours to 30 days |  |  |  |  |  |  |  |  |  |  |
|  |  | > 30 days |  |  |  |  |  |  |  |  |  |  |