SDS-PAGE BUFFERS

30:0.4 Acrylaminde/bis-acrylaminde Stock

[Final]			Mr/[Stock]		     	 /100ml

30% (w/v) acrylamide 		    71.08		  	   30.0 g

0.4% (w/v) bis-acrylaminde	  154.17			0.4 g

• Dissolve in ~70 mL MilliQ water by stirring until dissolved.

• DEIONIZE with MIXED BED RESIN (Sigma, M-8032) and filter through 0.22 m filter.

• Store protected from light in brown glass bottle.

4X CONCENTRATED Running Gel Buffer, pH 8.8

[Final]				Mr/[Stock]		     	 /100ml

1.5 M Tris-HCl, pH 8.8 		    121.1		  	   18.17 g

0.4% SDS	    		    288.38			0.4 g

4X CONCENTRATED Stacking Gel Buffer, pH 6.8

[Final]				Mr/[Stock]		     	 /100ml

0.5 M Tris-HCl, pH 8.8 		    121.1		  	     6.06 g

0.4% SDS	    		    288.38			0.4 g

10X CONCENTRATED SDS-PAGE Reservoir Buffer

[Final]		[10X]		     	 Mr		         /1 l		/ 2 L

25 mM Tris			0.25 M	           121.1	  	        30.3 g	            60.6 g

192 mM Glycine	         1.92 M	            75.07                     144.1 g                288.2 g

0.1% SDS	    	         1.0%                  —                          10.0 g	 	20.0 g

10X CONCENTRATED Immunoblotting TRANSFER Buffer

[Final]		    	    [10X]		  Mr		         /1 l		/2  l

15.6 mM Tris-HCl	    	  0.156 M	  	121.1		        18.9 g	 	37.8 g

120 mM glycine    	           1.2 M	         	          75.07	   90.0 g                180.0 g

10 X CONCENTRATED Tris-buffered saline + Tween (TBST)   pH 7.5

[Final]	              [10X]	           Mr                         / l l	 		/2 l

10 mM Tris-HCl, pH 7.5               0.1 M                       121.1                       12.1 g			24.2 g

150 mM NaCl	              	             1.5 M   	           58.44                     87.7 g	            175.3 g

0.1% Tween 20	              1.0%	              —                       10.0 g		       	20.0 g

SDS-PAGE RECIPIES

6.5% gel  (8 ml)					11.0 % gel (20 ml)

acrylamide		1.7 ml			acrylamide			7.4 ml

runnning buffer  (pH 8.8)	2.0 ml			running buffer (pH 8.8) 	5.0 ml

H2O		4.3 ml			H2O				7.6 ml

Temed		4µl			Temed				12 µl

APS		48 µl			APS				120 µl

6.5% gel  (20 ml)					12.5 % gel (20 ml)

acrylamide		4.35 ml			acrylamide			8.4 ml

runnning buffer  (pH 8.8)	5.0 ml			running buffer (pH 8.8) 	5.0 ml

H2O		10.65 ml			H2O				6.6 ml

Temed		12µl			Temed				12 µl

APS		120 µl			APS				120 µl

8.0% gel  (20 ml)					14.0% gel (20 ml)

acrylamide		5.4 ml			acrylamide			9.4 ml

runnning buffer  (pH 8.8)	5.0 ml			running buffer (pH 8.8) 	5.0 ml

H2O		9.6 ml			H2O				5.6 ml

Temed		12µl			Temed				12 µl

APS		120 µl			APS				120 µl

10.0% gel  (20 ml)					4.0% STACKING GEL (5 ml)

acrylamide		6.7 ml			acrylamide			0.67ml

runnning buffer  (pH 8.8)	5.0 ml			stacking buffer (pH 6.8) 	1.25 ml

H2O		8.3 ml			H2O				3.08ml

Temed		12µl			Temed				10 µl

APS		120 µl			APS				60 µl