PREPARATION OF RAT BRAIN SYNAPTOSOMES

Preparation #       (                )

• SYNAPTOSOME HOMOGENIZATION BUFFER  (refractive index = 1.          )

[Final]		              Mr		  /l		/500 ml

5 mM Hepes-KOH, pH 7.4	238.3		1.19g		   0.6g

320 mM sucrose		342.3		109.5g	   54.8g

• 6% (w/v) FICOLL 400 IN HOMOGENIZATION BUFFER  (ref. index = 1.          )

[Final]				[Stock]		 /100 ml

5 mM Hepes-KOH, pH 7.4		238.3			  up to

320 mM sucrose			342.3			 100 ml

6% (w/v) Ficoll 400		100%			   6.0g

• 9% (w/v) FICOLL 400 IN HOMOGENIZATION BUFFER  (ref. index = 1.          )

[Final]		           [Stock]		 /50 ml

5 mM Hepes-KOH, pH 7.4		238.3			   up to

320 mM sucrose			342.3			  100 ml

9% (w/v) Ficoll 400		100%			   4.5g

• 13% (w/v) FICOLL 400 IN HOMOGENIZATION BUFFER  (ref. index = 1.          )

[Final]			[Stock]		 /100 ml

5 mM Hepes-KOH, pH 7.4		238.3			  up to

320 mM sucrose			342.3			 100 ml

13% (w/v) Ficoll 400		100%			   13.0g

• All procedures carried out at 0-4°C.

• 	4 male Sprague-Dawley rats (SASCO, 11/22/97) used. Animal killed by CO2 	inhalation, followed by decapitation. Brains rapidly removed and the myelin-	rich white portions of the central brain and brain stem crudely dissected away 	from the cortex and cerebellum with scissors. Dissected "gray matter" placed into 	a beaker of ice-cold homogenization buffer in ice.

• Weight of beaker before =              g

after =              g

g   TOTAL RAT BRAIN USED

• Brains first homogenized in about 5 volumes of ice-cold homogenization buffer.

• ~        ml homogenization buffer added to a 50 ml homogenizer on ice.

• Dissected brains added and homogenized with 8 up-and-down strokes of the 	"loose" Teflon pestle of a Potter-Elvehjem homogenizer rotating at a setting 	"10.0"  (max) on Deuels motor until.

• Homogenate decanted into a 100 ml cylinder on ice.

• TOTAL VOLUME OF HOMOGENATE =          ml

• Decanted into a polycarbonate Sorvall tube on ice.

• Centrifuged at ~3,020 gmax (5,000 rpm, JA-20 rotor) 4°C, 2 min.

• Cream/white-colored turbid supernatant decanted off the large trilaminate pellet  	[bottom gray/pink-colored granular layer, middle red erythrocyte-containing 	layer and upper fine white membrane layer] into a cold measuring cylinder.

• Pellet resuspended in ~30 ml ice-cold homogenization buffer, 15 ml 0.2 M PMSF 	added, and homogenized with 3 manual up-and-down strokes of the Teflon 	pestle.

• Centrifuged at ~3,020 gmax (5,000 rpm, JA-20 rotor) 4°C, 2 min.

• Cream/white-colored turbid supernatant decanted off the large trilaminate pellet  	and pooled in the cold cylinder with the first supernatant 1.

• TOTAL VOLUME OF SUPERNATANT 1 =         ml.

• Decanted into 2X polycarbonate Sorvall tubes on ice.

• Centrifuged at ~ 13,000 gmax (11,000 rpm, JA-20 rotor) 4°C, 12 min.

• Turbid, pale-pink/cream-colored supernatants decanted into a cold 100 ml               	measuring cylinder.

• TOTAL VOLUME OF SUPERNATANT 2 =         ml.

• Bilaminate pellet 2 [large white/cream-colored upper portion with a 	greenish/brown-colored lower region] resuspended in residual supernatant.

• Volume of resuspended pellet adjusted to ~70 ml with ice-cold homogenization 	buffer in 2X polycarbonate Sorvall tubes on ice.

• Centrifuged at ~ 13000 gmax (11,000 rpm, JA-20 rotor) 4°C, 12 min.

• Opalescent supernatants aspirated and discarded.

• White/greenish-colored bilaminate pellets resuspended by swirling in residual 	supernatant avoiding the more tightly packed greenish/brown mitochondrion-	rich lower portion.

• Resuspended membranes transferred to a 50 ml polypropylene tube and volume adjusted to ~ 18 ml with ice-cold homogenization buffer.

TOTAL VOLUME OF RESUSPENDED P2' PELLET =       ml.

• 6X Ficoll/sucrose gradients prepared in ultraclear SW41 tubes.

• Into each tube on ice:

4 ml 13% Ficoll/sucrose

1 ml 9% Ficoll/sucrose

4 ml 6% Ficoll/sucrose

~ 3 ml resuspended washed pellet (P2')

• Centrifuged at ~ 86,800 gmax (22,500 rpm, SW41 rotor) 4°C, 35 min.

• After centrifugation, a fluffy white-colored myelin membrane band is found at 	the load/6% Ficoll interface, cream/white-colored synaptosome-enriched 	membrane layers found at the 6%/9% and 9%/13% Ficoll interfaces and a 	greenish-colored, mitochondrion-rich pellet found at the bottom of each tube.

• Load zone, myelin band and the 6% Ficoll layer of each gradient aspirated with a 	Pasteur pipette and discarded.

• Synaptosome-enriched bands aspirated with a Pasteur pipette and polled into a polycarbonate Sorvall tube on ice.

• TOTAL VOLUME OF GRADIENT-PURIFIED SYNAPTOSOMES =         ml.

• Diluted to a final volume of ~ 100 ml (i.e. 5-fold) with ice-cold homogenization buffer in 3X polycarbonate Sorvall tubes on ice.

• Centrifuged at ~ 27,200 gmax (15,000 rpm, JA-20 rotor) 4°C, 12.5 min.

• Clearish supernatants aspirated and discarded.

• Synaptosome-rich pellet resuspended in a minimal volume of homogenization buffer on ice.

• TOTAL VOLUME OF RESUSPENDED SYNAPTOSOMAL PELLET =          ml.

• Protein determination of fractions from synaptosome prep #         :

• MiniBradford assay using 0–8 μg BSA as standards.

Standard Curve  		A595			  x

0 μg BSA		0.000		    	0.00

1 μg BSA

2 μg BSA

4 μg BSA

8 μg BSA

SAMPLE	         VOLUME	         A595	     x 	           μg    	           [  ]

Homogenate	              0.2 μl

Supernatant 1                0.2 μl

Supernatant 2                0.2 μl

(Supernatant 3)*            0.5 μl

Washed pellet 2 (P2')        1.0 μl

Synaptosomes #   	        μl

* prepared by centrifuging a small aliquot of supernatant 2 at 245,000 (70,000 rpm, TLA-100.3 rotor) 4°C, 20 min in a Beckman microfuge tube.